PROJECT PROPOSAL

Microgravity Validation of a Vibration and

Electrical Harmonic Diagnostic Model

for DC Motors

Single-Axis vs Multi-Axial Model Comparison

March 2026

# Table of Contents

Table of Contents	2

1. Context and Objective	3

2. The Two Models	3

2.1 Model A - “Single-Axis” (Baseline)	3

2.2 Model B - “Multi-Axial” (Robust Candidate)	3

3. Ground Phase - Training	4

3.1 Motors and Faults	4

3.2 Sensors	4

3.3 Acquisition Protocol	4

3.4 Processing Pipeline (Common)	4

3.5 Ground Validation	4

4. Flight Phase - Microgravity Experiment	6

4.1 Onboard Hardware	6

4.2 Flight Protocol	6

4.3 Data Collected	6

5. Return Phase - Comparative Analysis	7

5.1 Primary Analysis - Blind Test	7

5.2 Secondary Analysis - Domain Shift	7

5.3 Tertiary Analysis - Transfer and Adaptation	7

5.4 Expected Results Matrix	7

6. Deliverables	8

7. Mass / Power Budget	8

8. Summary of A/B Design Value	8

# 1. Context and Objective

Problem Statement: Predictive diagnostic models trained on the ground implicitly incorporate gravitational effects (axial bearing loads, shaft deflection, weight-related structural resonances). The question is: does a model trained at 1g remain reliable at 0g?

Central Hypothesis: A model trained across multiple gravitational orientations (multi-axial) learns features that are more gravity-invariant than a model trained in a single orientation, and will therefore be more robust in microgravity.

Objective: Compare two embedded training strategies and quantify the robustness gain provided by ground-based orientation diversity.

# 2. The Two Models

## 2.1 Model A - “Single-Axis” (Baseline)

- Training orientation: Horizontal only (shaft parallel to ground)

- Data: ~100 acquisitions per configuration (fault × load × speed)

- Hypothesis: Gravity vector remains constant → model implicitly encodes gravitational signature

- Size: < 50 KB (INT8, CyanMycelium)

## 2.2 Model B - “Multi-Axial” (Robust Candidate)

- Training orientations: 6 orientations (0°, +90°, −90°, ±45°, 180°)

- Data: ~100 acquisitions × 6 orientations per configuration

- Hypothesis: Orientation diversity forces the model to learn gravity-vector-invariant features

- Size: < 50 KB (INT8, CyanMycelium)

Identical architecture for both models - only the training data differs. This strictly isolates the effect of the training strategy.

# 3. Ground Phase - Training

## 3.1 Motors and Faults

20+ identical brushed DC motors (e.g., Mabuchi RS-385). Injected faults:

- D0 - Healthy (reference)

- D1 - Rotor imbalance (added mass)

- D2 - Bearing fault (inner race micro-scratch)

- D3 - Brush wear (pre-worn brushes)

- D4 - Shaft eccentricity (controlled misalignment)

## 3.2 Sensors

| Signal | Sensor | Sampling Rate |
| --- | --- | --- |
| 3-axis vibration | ADXL355 | 4 kHz |
| Motor current | Shunt + 16-bit ADC | 10 kHz |
| Motor voltage | Divider + ADC | 10 kHz |
| Temperature | NTC thermistor | 1 Hz |
| RPM | Reflective optical encoder | Continuous |

## 3.3 Acquisition Protocol

Model A: each motor in horizontal position only. 3 load profiles × 3 speed profiles × 100 repetitions = 900 acquisitions/motor.

Model B: each motor in all 6 orientations. 3 loads × 3 speeds × 6 orientations × 100 repetitions = 5,400 acquisitions/motor.

## 3.4 Processing Pipeline (Common)

- DSP: FFT, spectral envelope, STFT

- Features: fundamental harmonics + sidebands, vibration RMS, kurtosis, crest factor, MCSA

- Model: identical network architecture, quantized INT8 via CyanMycelium

- Output: fault class + confidence score

## 3.5 Ground Validation

Model A:

- Cross-validation (leave-one-motor-out) on horizontal only

- Test on 5 unseen orientations

- Target metric: Accuracy > 95% on horizontal

Model B:

- Cross-validation (leave-one-motor-out) on all orientations

- Target metric: Accuracy > 95% on mixed orientations

Critical ground test: Model A performance on untrained orientations. If A already collapses between horizontal and vertical, we have a strong indicator before flight.

# 4. Flight Phase - Microgravity Experiment

## 4.1 Onboard Hardware

| Element | Qty | Notes |
| --- | --- | --- |
| Compute board (STM32H7 / RP2040) | 2 | Nominal + spare |
| Acquisition board (ADC + cond.) | 2 | ADC + signal conditioning |
| Healthy motors (D0) | 4 | Baseline |
| D1 motors (imbalance) | 2 |  |
| D2 motors (bearing) | 2 |  |
| D3 motor (brush wear) | 1 |  |
| D4 motor (eccentricity) | 1 |  |
| Industrial SD storage | 2 | Raw data storage |

10 motors total, numbered, fault mapping in sealed envelope.

## 4.2 Flight Protocol

For each motor:

- Mount on support with standard fixture

- Automated sequence: ramp → steady state → ramp → stop

- Dual simultaneous inference: Model A and Model B predict class + confidence

- Raw data recorded to SD

- Minimum 3 repetitions per motor

Estimated duration: ~2h (mostly automated)

## 4.3 Data Collected

Per motor and per run:

- Raw data: 3-axis vibration (4 kHz), current (10 kHz), voltage (10 kHz), RPM, temperature

- Inferences: Model A prediction (class, confidence, latency), Model B prediction (class, confidence, latency)

- Metadata: motor_id, run_id, timestamp

# 5. Return Phase - Comparative Analysis

## 5.1 Primary Analysis - Blind Test

Compare for both models: overall 0g accuracy, per-class F1 score, average confidence, false negative rate.

Expected result: B > A across all metrics at 0g.

| Metric | Model A | Model B | Delta |
| --- | --- | --- | --- |
| Overall 0g accuracy | ? | ? | ? |
| Per-class F1 | ? | ? | ? |
| Average confidence | ? | ? | ? |
| False negatives | ? | ? | ? |

## 5.2 Secondary Analysis - Domain Shift

- Ground/flight spectral comparison for each fault type

- Quantify the percentage of features affected by gravity

- Identify frequency bands most sensitive to the gravity vector

- Correlation: do features that change between ground orientations match those that change at 0g?

## 5.3 Tertiary Analysis - Transfer and Adaptation

- How much 0g data is needed to adapt A? (incremental fine-tuning with N = 1, 5, 10, 20 samples)

- Does B need adaptation? (direct test, then fine-tuning if necessary)

- Is ground multi-axial a good proxy for 0g? (feature distribution comparison)

- Can we build a “gravity-invariant” model? (domain adversarial training on orientation)

## 5.4 Expected Results Matrix

|  | Ground (horiz.) | Ground (multi) | Flight (0g) |
| --- | --- | --- | --- |
| Model A (single) | Excellent | Poor | Poor |
| Model B (multi) | Good | Excellent | Good |

Key question: does multi-orientation ground performance predict 0g flight performance?

# 6. Deliverables

| Deliverable | Format |
| --- | --- |
| Complete ground dataset (single + multi-axial) | HDF5 + JSON metadata |
| 2 trained models | ONNX + CyanMycelium binaries |
| Flight dataset (raw + dual inference) | HDF5 + JSON metadata |
| Comparative report A vs B | Publication |
| Space training strategy recommendations | Technical document |

# 7. Mass / Power Budget

| Parameter | Value |
| --- | --- |
| Total mass | < 2.5 kg |
| Peak power | < 15W |
| Volume | ~2U CubeSat equivalent |
| Candidate platforms | ISS (NanoRacks, ICE Cubes), parabolic flight, CubeSat |

# 8. Summary of A/B Design Value

Embedding both models on the same board and running them on identical raw data provides a perfectly controlled comparison.

Any performance difference is strictly attributable to the training strategy, not to hardware, flight conditions, or tested motors.